# Methodology for Firm-Level MACC Scaling

### 1. Overview

This methodology explains how a sector-level reference Marginal Abatement Cost Curve (MACC) is scaled to generate firm-level MACCs.

### 2. Key Variables and Definitions

| Symbol | Description | Units |
| --- | --- | --- |
| Pf | Production of firm | units / year |
| Ef | Total emissions of firm | t CO₂ / year |
| EIf = Ef / Pf | Emission intensity | t CO₂ / unit |
| Ai(ref) | Abatement potential of intervention i (reference firm(sector)) | t CO₂ / year |
| Ci | Cost of intervention i | $ / t CO₂ |
| ΔEIi = Ai(ref) / Pref | Reduction in emission intensity per unit product | t CO₂ / unit |

### 3. Reference Firm/Sector  MACC

For each sector, define:

- Reference production Pref

- Reference emissions Eref

Baseline intensity:
EIref = Eref / Pref

Each intervention i reduces intensity by ΔEIi.
Total reduction across all measures:

Dref = Σ ΔEIi

The interventions are sorted by increasing cost Ci to form the reference MACC.

### 4. Mapping Firms to the Reference MACC

Each firm has its own production Pf and intensity EIf.
How we map it depends on whether it’s cleaner(lower-intensity) or dirtier(higher-intensity) than the reference.

#### Cleaner Firm (EIf < Dref)

Already achieved some early, low-cost measures.
The achieved portion is removed:

achieved = Dref - EIf

- Skip all interventions where ΔEIi ≤ achieved.

- If the skip stops part-way through a measure, keep only the remaining share:

ΔEIi(remain) = ΔEIi - achieved

#### Dirtier Firm (EIf > Dref)

Needs one extra intervention step to reach zero with the cost slightly less then cheapest intervention:

ΔEIadd = EIf - Dref
Cadd = Cmin - 10

The extra step is inserted to the left of the cheapest bar, then the full reference macc follows.

### 5. Abatement Potential Calculation

For each intervention i in firm f:

- Abatement potential: Ai(f) = Pf × ΔEIi (t CO₂ / yr)

### 6. Summary of Core Equations

| Equation | Description |
| --- | --- |
| EIref = Eref / Pref | Reference intensity |
| ΔEIi = Ai(ref) / Pref | Per-unit intensity reduction |
| Ai(f) = Pf × ΔEIi | Firm-level abatement potential |
| achieved = Dref - EIf | Already implemented reduction |
| ΔEIadd = EIf - Dref | Extra reduction for Above-reference firms |

# Example: Aluminium Sector (Reference and Two Firms)

### Reference firm data

Sector: Aluminium
Production (P_ref) = 10,000 units per year
Emissions (E_ref) = 20,000 tCO₂ per year
Emission intensity:

EI_ref = E_ref / P_ref = 20,000 / 10,000 = 2.0 tCO₂ per unit

### Reference MACC interventions

| ID | Intervention Name | Cost (USD/tCO₂) | Abatement Potential (tCO₂/yr) |
| --- | --- | --- | --- |
| A1 | EE: maintenance & tuning | −300 | 2,000 |
| A2 | EE: drives & controls | −150 | 2,500 |
| A3 | Waste-heat recovery | 50 | 3,000 |
| A4 | Motor/insulation upgrades | 120 | 3,500 |
| A5 | Process optimization | 250 | 4,000 |
| A6 | Fuel switch (alt-fuel) | 500 | 3,000 |
| A7 | RE electrification | 900 | 2,000 |

Convert to per-unit intensity reductions:

ΔEI_i = AbatementPotential_i / P_ref

ΔEI = [0.20, 0.25, 0.30, 0.35, 0.40, 0.30, 0.20]

Total reference reduction:

D_ref = Σ ΔEI_i = 2.00

## Firm AL-001 (Below-reference firm)

Production (P_f) = 8,000 units/year
Emission intensity (EI_f) = 1.5 tCO₂/unit

lower-intensity because EI_f < D_ref
Already achieved:

achieved = D_ref - EI_f = 2.00 − 1.50 = 0.50

Skip the first two cheapest measures (A1 + A2 = 0.45) and a partial of A3 (0.05).
Remaining part of A3 = 0.30 − 0.05 = 0.25.

Now include the rest of the measures fully.

### Firm AL-001 Resulting MACC

| order | ID | Intervention Name | Cost | ΔEI | Abatement Potential (tCO₂/yr) | Cumulative Abatement (tCO₂/yr) | Post-Intensity (tCO₂/unit) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | A3 | Waste-heat recovery | 50 | 0.25 | 2,000 | 2,000 | 1.25 |
| 2 | A4 | Motor/insulation upgrades | 120 | 0.35 | 2,800 | 4,800 | 0.90 |
| 3 | A5 | Process optimization | 250 | 0.40 | 3,200 | 8,000 | 0.50 |
| 4 | A6 | Fuel switch (alt-fuel) | 500 | 0.30 | 2,400 | 10,400 | 0.20 |
| 5 | A7 | RE electrification | 900 | 0.20 | 1,600 | 12,000 | 0.00 |

### Verification for Firm AL-001

Abatement Potential = P_f × ΔEI_i
= 8,000 × [0.25, 0.35, 0.40, 0.30, 0.20]
= [2,000, 2,800, 3,200, 2,400, 1,600]

Total abatement = 12,000 tCO₂/yr
Baseline emissions = P_f × EI_f = 8,000 × 1.5 = 12,000 tCO₂/yr 
Post-intensity = 1.50 → 1.25 → 0.90 → 0.50 → 0.20 → 0.00

## Firm AL-002 (Above-reference firm)

Production (P_f) = 11,000 units/year
Emission intensity (EI_f) = 2.2 tCO₂/unit

higher-intensity firm because EI_f > D_ref.
Need additional reduction:

ΔEI_add = EI_f - D_ref = 2.2 − 2.0 = 0.2

New added measure cost:

C_add = C_min − 10 = −300 − 10 = −310

This additional intervention is added at the beginning of the cost order.

| order | ID | Intervention Name | Cost | ΔEI | Abatement Potential (tCO₂/yr) | Cumulative Abatement (tCO₂/yr) | Post-Intensity (tCO₂/unit) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | A0 | eff_improvement | −310 | 0.20 | 2,200 | 2,200 | 2.00 |
| 2 | A1 | EE: maintenance & tuning | −300 | 0.20 | 2,200 | 4,400 | 1.80 |
| 3 | A2 | EE: drives & controls | −150 | 0.25 | 2,750 | 7,150 | 1.55 |
| 4 | A3 | Waste-heat recovery | 50 | 0.30 | 3,300 | 10,450 | 1.25 |
| 5 | A4 | Motor/insulation upgrades | 120 | 0.35 | 3,850 | 14,300 | 0.90 |
| 6 | A5 | Process optimization | 250 | 0.40 | 4,400 | 18,700 | 0.50 |
| 7 | A6 | Fuel switch (alt-fuel) | 500 | 0.30 | 3,300 | 22,000 | 0.20 |
| 8 | A7 | RE electrification | 900 | 0.20 | 2,200 | 24,200 | 0.00 |

### Verification for Firm AL-002

Abatement Potential = P_f × ΔEI_i
= 11,000 × [0.20, 0.20, 0.25, 0.30, 0.35, 0.40, 0.30, 0.20]
= [2,200, 2,200, 2,750, 3,300, 3,850, 4,400, 3,300, 2,200]

Total abatement = 24,200 tCO₂/yr
Baseline emissions = P_f × EI_f = 11,000 × 2.2 = 24,200 tCO₂/yr  
Post-intensity = 2.20 → 2.00 → 1.80 → 1.55 → 1.25 → 0.90 → 0.50 → 0.20 → 0.00